### **Threads in a CPU**

* In a CPU, a thread is a *software-managed sequence of instructions* that runs on a physical core.

* CPUs usually have only a few cores (4, 8, 16, etc.), and each core can run 1 or 2 threads at a time using *hyper-threading*.

* CPU threads are designed for **complex, general-purpose tasks** like running the operating system, web browsing, or handling databases.

* Each CPU thread gets a lot of resources: big caches, strong control logic, and high performance per thread.

**Example:**

If your CPU has 8 cores with hyper-threading, it can run 16 threads in parallel. But each thread is powerful and good at handling complicated instructions.

---

### **Threads in a GPU**

* In a GPU, threads are **much smaller and lighter** than CPU threads.

* A GPU can have **thousands of threads running at once** because it’s built for parallel workloads.

* GPU threads are grouped into **warps** (NVIDIA) or **wavefronts** (AMD), where many threads execute the same instruction on different pieces of data.

* GPU threads are simple and don’t have as many resources individually — but when thousands work together, they can process huge amounts of data quickly.

* They are mainly used for **parallel tasks** like graphics rendering, simulations, machine learning, and scientific computing.

**Example:**

A modern GPU can run 10,000+ threads at the same time. Each thread may only handle a small piece of an image, but together they can render an entire frame of a game in milliseconds.

---

### **Main Difference**

* **CPU threads** → Few in number, powerful, handle complex sequential tasks.

* **GPU threads** → Thousands, lightweight, handle massively parallel tasks where the same work is repeated on lots of data.